[image: image1.png]Video Storm LIC
Wwww. vi deo- storm. com







Model HMX42



HDMI matrix switch
Overview:

This product is a full featured HDMI matrix switch.  It is most commonly used to independently distribute video & audio to several different viewing areas from a set of common sources.  HMX44 provides 2 output zones from 4 common sources.

The matrix switch has 4 control modes to choose from:  
· Manual control from the front buttons

· IR control by remote control

· Serial control via the RS-232 port (db9)
· Integrated control from the CMX matrix switch

Unit includes
· HMX42 HDMI matrix switch
· Owners manual
· Remote control
· IR detector
· 5V wall power supply
· 1 year warranty
Installation
Please use the following steps to install and set up your HMX42.
1. HDMI inputs:  Connect your HDMI sources to the HDMI input jacks labeled 1-4.  You can also use DVI sources with a DVI-HDMI cable.
2. HDMI outputs:  Connect your HDMI displays to the HDMI input jacks labeled 1-2.  You can also use DVI displays with a DVI-HDMI cable.

3. Digital audio:  Connect your receivers to the digital audio output jacks labeled 1-2.  You can use either coax or toslink, both are active.
4. If using the IR detector, connect it to the rear jack labeled IR.  Place the IR detector in location with line of sight to your remote.

5. RS-232:  If connecting to the CMX matrix, connect a DB9 male to female normal modem cable from the RS-232 jack on HMX42 to the “XP” jack on the CMX matrix
6. Connect the supplied wall transformer to a properly grounded power outlet.  Plug into DC input of the HMX42.
7. Turn on the power switch; you should see the LEDs turn on.

Controlling the HMX42
The CMX44 can be controlled by 4 methods:  front panel switch, infrared remote, RS-232 serial, or by connection to the CMX matrix.  All methods can access all features and modes.
The front panel buttons can select which input is routed to each output.  Each selection button controls one output.  Press the button to toggle through input selections.  The LEDs will indicate which input is currently selected.
The IR remote and RS-232 interface can select outputs and inputs directly.  If using the RS-232 interface, please refer to the programming guide on our web site.   The IR interface is controlled by the include remote control.  Simply press the button corresponding to the input / output combination you wish to activate.
When controlling the HMX42 through the CMX matrix switch, you do not need to use the front panel buttons or IR remote for the HMX42.  All control will be handled by the CMX matrix.  

On the CMX matrix, set the “CF2F” config setting to “CF2O”.  This will enable the CMX to automatically detect the HMX switch.  This setting is saved in flash, so you do not need to repeat it.  The HMX outputs will now be available as output type “H”, with 2 outputs and 4 inputs selectable (for the HMX42).  If the H outputs do not appear on the CMX control menu, please cycle the power on the HMX42.  

Specifications
· 4 HDMI inputs  (version 1.3)
· 2 HDMI outputs (version 1.3)
   Tested to 1080p resolution
   1.65Gbps maximum signal rate
   All output sets individually controllable/switchable
   Output selection memory to retain state during power loss
· 2 Digital audio outputs
    Carries the digital audio present on the selected HDMI input  (HDMI output also carries this)
    Both coax and toslink connectors present and active
· IR receiver (front) 
· IR wired receiver (rear)
· RS-232 control input (DB9 female)

· Housing 
    Black steel rack mountable housing   5.1" x 10.5" x 2.2" 
Warranty

Video Storm LLC products are covered by our manufacture’s warranty.   Our warranty covers parts and labor for 1 years following purchase.  In the unlikely event you receive an item that was not damaged in shipping but is defective please contact support@video-storm.com and we will repair or replace the item promptly.

