TMX RS-232 control guide

Revision 0.9.2

1/5/10

Overview

The RS-232 port on the TMX series switches is a flexible interface which can easily control all aspects of the device operation.  Many system integrators prefer RS-232 control over IR because it allows the controlling device to monitor the status of the TMX switch.  The TMX series switches also feature a feedthrough RS-232 port which can allow multiple devices to be controlled from a single RS-232 source.

Hardware specifics

The TMX switch uses a DB 9 female connector (shown below) for connection to the controller.

Only 3 pins on the connector are needed:

Pin 2:  TX  (from TMX to controller)

Pin 3:  RX  (from controller to TMX)

Pin 5:  Ground

The cable used should be a standard straight through RS-232 cable with a DB9 male plug to DB9 or 25 female plug (also known as a serial modem cable).

The TMX switch uses a DB 9 male connector (shown below) for the expansion port.

Only 3 pins on the connector are needed:

Pin 2:  RX  (from downstream to TMX)

Pin 3:  TX  (from TMX to downtstream)

Pin 5:  Ground

The cable used should be a standard straight through RS-232 cable with a DB9 female to DB9 or 25 male plug (also known as a serial modem cable).

Protocol settings

Baud rate : 9600

Data bits: 8

Stop bits: 1

Parity: None

Flow control: None

Commands

TMX will echo back all characters sent to it.  This is the easiest way to verify if your cable connection is correct.  The TMX does not add <lf> after any <cr> received, so if you are using windows Hyper-terminal you should change the default settings to allow line feed on carriage return.

All commands are terminated by <cr> (carriage return, ascii code 0xD).  NOTE:  all references to <cr> in this document mean the single ascii character NOT the four characters “<cr>”.  Commands can be forwarded to the expansion port by adding a prefix “/F” to each command (see section below).

Selecting input “00” will disable the selected output.

The following are the valid command sets for all TMX devices.  Different models can be differentiated by the controller by the status readback.

Command set for config controls:          NOTE, all config settings stored in FLASH

CF##<cr> :  Config control

1st # selects which control you will change

2nd # F control off, T turns control on

Control 1:  Clear config bits  (T runs clear, auto sets   back to F)

Control 2:  LED off

Control 3:  Power off

Control 4:  Advanced scheme enable

Control 5:  Force HDMI detect enable

Control 6:  Matrix auto refresh enable

Control 7:  Power on default setting save enable

Control 8:  Forward channel IR enable

Control 9:  Debug control 1  (leave as “F”)

STAT<cr> :  Request device status

STATLONG<cr> :  Request detailed device status

NAMEO## CCCCCCCCCCC<cr>:  Names output zone ## to given string

NAMEI## CCCCCCCCCCC<cr>:  Names input zone ## to given string

CFLENaabbb<cr>:  Set cable length for output aa to bbb meters

SDATEmmddyy<cr>:  Set Date to mm/dd/yy

STIMEhhmmss<cr>:  Set Time to hh:mm:ss  (military time)

BLIGHTxxx<cr>:  Set LED backlight to xxx (001-100)

LOCKx<cr>:  For x=T, lock front panel.  x=F, unlock front panel

This control is not stored in flash

Device always boots with front panel enabled

TDLIMITaabb ####### hhmmss hhmmss<cr>:

Time of day limit for source aa to zone bb

Can on be access between time1 and time2

# field is the day of the week mask (Sunday => Saturday)

Set 1 to enable on that day, 0 to disable

If aa or bb is = 00 then applies to all zones/inputs

If both = 00 all TDlimits erased

TLIMITaabb # hhmmss <cr>:

Time elapsed limit for source aa to zone bb

Can only be access for total time1 per day

# field is the day of the week mask (Sunday => Saturday)

Set 1 to enable on that day, 0 to disable

If aa or bb is = 00 then applies to all zones/inputs

If both = 00 all Tlimits erased

Faab<cr>:  Flash modes control

aa is a number 01-08 selects the flash slot to control

b is either S or R (S saves current config, R recalls saved config)

Zaahhhh<cr>:  Zone restriction control

aa is a number 00-max selects the zone

hhhh is a 4 digit hexadecimal number corresponding to the ZONE MASK

Each bit of the MASK allows you to restrict an input

0 = input restricted

1 = input allowed

The LSB of the word is input 0, the MSB is input 16

JTaa(D/A)<cr>:  Audio zone jack type

aa is a number 00-max selects the zone

D:  Digital audio output is used (best if zone supports multichannel audio)

A:  Analog stereo output is used

IRVaa(T/F)<cr>:  IR detector voltage

aa is a number 00-max selects the zone

F:  5V supply (default)

T: 12V supply

AV Command set:

NOTE:  Output 0 is the local RCA output.  Input 0 is disable output.

Vaabb(S)<cr> :  Video output control

aa is a number 00-MAX selects the desired output zone to control

bb is a number 00-12 selects the input board to switch the output to

S is optional, if sent the controller will attempt to select the S-Video input instead of the default component or composite (only used with cvbs/svideo input card)

Aaabb<cr> :  Audio channel output control

aa is a number 00-MAX selects the desired output zone to control

bb is a number 00-12 selects the input board to switch the output to

Audio volume Command set :

Gaabbb<cr> :  Audio output gain control

aa is a number 00-MAX selects the desired output zone to control

bbb is a number 00-255 selects the output gain

For bbb = 000 output is muted

Else gain(dB) = 31.5 – [0.5*(255-bbb)]

bbb = U   Volume steps up

bbb = D   Volume steps down

bbb = MT  Mute on

bbb = MF  Mute false

bbb = M  Mute toggle

Eaabbb<cr> :  Audio input gain control

aa is a number 01-MAX selects the desired input zone to control

bbb is a number 00-255 selects the input gain

For bbb = 128 input is nominal

Else signal = signal * bbb/128

Maabbb<cr> :  Audio output balance control

aa is a number 00-MAX selects the desired input zone to control

bbb is a number 00-255 selects the balance setting

For bbb = 128 input is nominal

if (bbb>128) then right_vol=right_vol*(256-bbb)/128

else left_vol=left_vol*(bbb)/128

Baabbb<cr> :  Audio output bass control

aa is a number 00-MAX selects the desired input zone to control

bbb is a number 00-255 selects the input gain

For bbb = 128 input is nominal

Bbb is gain/atten applied to LF filter

Taabbb<cr> :  Audio output treble control

aa is a number 00-MAX selects the desired input zone to control

bbb is a number 00-255 selects the input gain

For bbb = 128 input is nominal

Bbb is gain/atten applied to HF filter

Laab<cr> :  Audio output loudness compensation control

aa is a number 00-MAX selects the desired input zone to control

b is either T or F to enable or disable loudness compensation

Daabbb<cr> :  Audio delay control

aa is a number 00-MAX selects the desired output zone to control

bbb is a number 00-255 selects the audio delay

For bbb = 000 output has zero added delay

Else delay(ms) = TBD

CHIME##tt<cr>:  Activate door chime      (placeholder for future firmware)

The chime command will temporarily switch audio on all outputs to input zone ## for tt seconds.  If ## is 00, then it will instead play the default internal chime.

Command set (HDMI):        (if supported HDMI switch attached via rs232 expansion)

Haabb<cr> : HDMI output control

aa is a number 01-16 selects the desired output to control

bb is a number 00-16 selects the input to switch the output to

Response strings

TMX devices have these possible response strings:

- ACKNOWLEDGE   (OK<cr>)   :  This is sent to indicate the previous command was executed

- STATUS    This is sent whenever explicitly requested by STAT command

- NOT ACKNOWLEDGE  (NAK<CR>) This is returned when an invalid command is received

- ACKNOWLEDGE for volume (OK:###(M)<cr>) :  When sending a volume command, the CMX will return the current setting after the OK.  If the “M” is present, that output is currently muted.

The format of the status response is:

Video Storm LLC TMX##### switch (with HDMI##-##)<cr><cr>

Version ### <cr>

Date mm/dd/yy    Time hh/mm/ss<cr>

V01bb(S)<cr>

V02bb(S)<cr>

(V03-Vmax)		Same as above

B01bb(A/D)<cr>

(B02-Bmax)

G01ddd(M)<cr>

(G02-Gmax)

E01ddd<cr>

(E02-Emax)

H01bb<cr>

(H02-Hmax)

CF2#<cr>

CF3#<cr>

CF4#<cr>

CF5#<cr>

CF6#<cr>

CF7#<cr>

CF8#<cr>

CF9#<cr>

(Next will be any saved blocks, time limits, day limits)

(these will be output in the format of the command which sets them)

The first line will give information on the model number and if there is a support HDMI matrix attached.  The numbers following TMX indicate the model number.  The numbers following the HDMI string are the number of outputs and inputs on the attached HDMI matrix.  If no HDMI matrix is attached, the HDMI string in parenthesis will not be sent.

The format of the status strings is the same as the command strings before.  V is video, B is audio channel, G is audio gain, H is HDMI,  and CF is config.

“bb” denotes the input selected (0-12).  For audio gain “ddd” denotes the gain setting (0-255).

Example 1:  V0304  =>  Component video output zone 3 has input board 4 selected

The config settings are interpreted as follows:

C2T || C2F	LED on when F

C3T || C3F	Power on when F

C4T || C4F	User mode when F, Advanced mode when T

C5T || C5F	HDMI detect forced on when T

C6T || C6F	Matrix auto refresh on when T

C7T || C7F	Power up default save on when T

C8T || C8F	Debug setting 1

C9T || C9F	Debug setting 2

Expansion port

The expansion port allows three possible connections:

- Connecting multiple TMX devices together to be controlled by a single rs-232 port.

- Connecting TMX supported HDMI matrix switches which will be controlled by the TMX device

- Connecting other generic rs-232 controlled equipment which will be controlled by the host controller in pass through mode.

You can combine uses 1 & 2 or 1& 3.  However, the generic equipment or HDMI switch must be connected to the last TMX unit and therefore terminates the chain.

In order to control the connected equipment, the host controller must know which devices are connected and their ordering.  A simple algorithm to detect this is shown in the last section (Recommended controller driver protocol).  Otherwise it can be hard coded if the configuration will not change.

The devices in the rs-232 should be numbered 0 to X-1, where X is the number of attached devices.  Device 0 is addressed using the normal protocol above.  All other devices are addressed by adding a prefix to their command string.  The FORWARD_COMMAND prefix is “/F” (two ascii characters).  When a device receives this string immediately after a <cr>, it will forward all the following characters to the next device in the chain until it receives a <cr> (the <cr> will also be forwarded, but the first “/F” will not).  Therefore, you address each device as follows:

Device 0 :  normal commands

Device 1 :  “/F” and then normal command  (terminated by single <cr>)

Device 2 :  “/F/F” and then normal command  (terminated by single <cr>)

Same upto X-1

If present, the last HDMI device does not need to be directly controlled.  It will be controlled by the last TMX in the chain.

IR distribution control

The TMX allows control of IR distribution over the RS232 (or Ethernet) port.  Currently there are two main functions.  The first is the ability to report received back channel IR commands out over the rs232 port to an external controller (IR-RX).  The second is the ability to send generic IR commands from an external controller (via rs232) to either the sources or IR forward channel (IR-TX).

IR-RX:

The TMX unit normally monitors all back channel IR data for TMX specific commands.  If the TMX is a TMX IR command (NEC code address 82h), then it processing the command.  If not, it routes the command to the selected source.

For TMX IR commands, command IDs 0-127 are used for switch control.  Command IDs 128-255 are routed to the RS232 port for an external controller to use (IR-RX commands).

When a IR-RX command is received, it sends the following RS232 data packet out the port (to both rs232 and Ethernet TCP-IP).

IRRXaabbcc<cr>null

aa = output zone command originates from (decimal)

bb = selected source for that output (decimal)

cc = command id received  (hex)

IR-TX:

The TMX unit allows sending any generic hex IR command from any of its IR outputs.  To send IR from an external host via the TMX, use the following command syntax.

IRTX###abb <cr>null

(hex IR code)<cr>null

### is the stored IR code number.  This is a placeholder for future enhancements.

If non-zero, no hex IR code is sent in the command

a is either “S” for source select or “Z” for zone select

bb is the source or zone number, this can be ** for all zones/sources

The hex IR code is sent after the first code.  This is the full HEX format code (spaces ignored).

Serial distribution control

Future TMXRX units will support forward channel or bidirectional RS232 commands.  The control command format will be as follows

SR-TX:

Serial commands may be sent to any output zone (with correct hardware) using the following format.

SRTX*****null

**** is the variable length serial word (may not contain NULL)

SR-RX:

If a serial command is received in any zone, it is routed to the external controller with the following format.  Note that commands may be split if the serial timing has delays.

SRRX****null

**** is the variable length serial word (may not contain NULL)