VMX RS-232 control guide

Revision 1.0

3/25/09

Overview

The RS-232 port on the VMX series switches is a flexible interface which can easily control all aspects of the device operation.  Many system integrators prefer RS-232 control over IR because it allows the controlling device to monitor the status of the VMX switch.

Hardware specifics

The CMX switch uses a 3.5mm stereo female connector for connection to the controller.

Connector pinout:

TIP:  TX  (from CMX to controller)

Ring:  RX  (from controller to CMX)

Sleeve:  Ground

Please use the DB9 female to 3.5mm stereo cable which can be purchased from Video Storm.

Protocol settings

Baud rate : 9600

Data bits: 8

Stop bits: 1

Parity: None

Flow control: None

Commands

VMX will echo back all characters sent to it.  This is the easiest way to verify if your cable connection is correct.  The VMX does not add <lf> after any <cr> received, so if you are using windows Hyper-terminal you should change the default settings to allow line feed on carriage return.

All commands are terminated by <cr> (carriage return, ascii code 0xD).  NOTE:  all references to <cr> in this document mean the single ascii character NOT the four characters “<cr>”.  Commands can be forwarded to the expansion port by adding a prefix “/F” to each command (see section below).

Selecting input “00” will disable the selected output.

The following are the valid command sets for all VMX devices.  Different models can be differentiated by the controller by the status readback.

Command set for config controls:

CF##<cr> :  Config control

1st # selects which control you will change

2nd # F control off, T turns control on

Control 1:  Clear ZeroKey IR codes & config bits  (T runs clear, auto sets   back to F)

Control 2:  LED off

Control 3:  Power off

Control 4:  Reserved

Control 5:  Debug control 1  (leave as “F”)

Control 6:  Debug control 2  (leave as “F”)

STAT<cr> :  Request device status

AV Command set:

Vaabb<cr> :	Component video output control

aa is a number 01-16 selects the desired output to control

bb is a number 00-16 selects the input to switch the output to

Response strings

VMX devices have two possible response strings:

- ACKNOWLEDGE   (OK<cr>)   :  This is sent to indicate the previous command was executed

- STATUS    This is sent whenever explicitly requested by STAT command

- NOT ACKNOWLEDGE  (NAK<CR>) This is returned when an invalid command is received

The format of the status response is:

Video Storm LLC VMX##### switch <cr>

Version 1.X<cr><cr>

V01bb<cr>

V02bb<cr>

(V03-Vmax)		Same as above

CF2#<cr>

CF3#<cr>

CF4#<cr>

CF5#<cr>

CF6#<cr>

“bb” denotes the input selected (0-16).  For digital audio, the “c” is D for digital coax or O or digital optical.  For audio gain “ddd” denotes the gain setting (0-255).

Example 1:  V0304  =>  Component video output 3 has input 4 selected

Example 2:  C16O =>  Coax audio output 1 has toslink audio input 6 selected

The config settings are interpreted as follows:

C2T || C2F	LED on when F

C3T || C3F	Power on when F

C4T || C4F	User mode when F, Advanced mode when T

C5T || C5F	Debug setting 1

C6T || C6F	Debug setting 2