Model VMX99

Ultra Wide Bandwidth HDTV

Matrix Switch

Overview:

This product is a high performance component video matrix switch.  It is most commonly used to independently distribute to several different viewing areas from a set of common sources.  VMX99 provides 9 output zones from 9 common sources.

The matrix switch has 4 control modes to choose from:

- Manual control from the front buttons

- IR control by remote control

- Serial control via the RS-232 port

- HTTP or TCPIP control via Ethernet port

Unit includes

- VMX99 hdtv matrix switch

- Owners manual

- 12V desktop power supply

- 3 year parts and labor warranty

- Optional rack mounting ears & screws

- Optional RS232 DB9 female to 3.5mm stereo cable

Warranty

Video Storm LLC products are covered by our manufacture’s warranty.   Our warranty covers parts and labor for 3 years following purchase.  In the unlikely event you receive an item that was not damaged in shipping but is defective please contact support@video-storm.com and we will repair or replace the item promptly.

Installation

Please use the following steps to install and set up your VMX99.

When looking at the rear of the VMX99, all inputs are on the left half and all outputs are on the right half.

- Component video:  Connect the component video sources to the input red/blue/green RCA jacks marked 1-9.  Connect the output cables to the output red/blue/green RCA jacks marked 1-9.  Be sure to match the cable colors to RCA jack color.

- RS-232:  If needed, connect your computer or controller to the DB9 jack labeled “Control” using the  DB9 female to 3.5mm stereo cable.

- Ethernet:  If needed, connect a standard Ethernet cable from your router to the Ethernet jack on VMX99.  If connecting directly to a computer, use a crossover cable instead.

- Connect the supplied wall transformer to a properly grounded power outlet.  Plug into DC input of the VMX99.

- Press one of the front panel buttons; you should see the LED display turn on.

Controlling the VMX99

The VMX99 can be controlled by 4 methods:  front panel switch, infrared remote, RS-232 serial, or Ethernet.  You may use any or all methods in any combination.  All methods can access all features and modes.

The left front panel button controls the output selection (left two LED digits).  Each press will cycle through the available outputs.  The right button controls the input selection (right two LED digits).  Each press will cycle through the available inputs.  Pressing and holding the left button for 3 or more seconds will jump the output selection to the next connector type when released.  Pressing and holding the right button for 3 or more seconds is only used in Zerokey programming mode, config mode, or for quick select mode.

NOTE: Output needs to be selected before setting the input.

Quick select can be used to easily send a single input to all outputs.  To use quick select, simply select output 1 using the left button, toggle to the input desired using the right button, and then press and hold the right button for 3 or more seconds.

Flash modes allow the user to save and later recall up to 8 different switch setups.  Flash modes are saved to flash memory, so are unaffected by power cycling.  To use the flash modes, select “Fx” (x is 1-8) using the left button (or IR).  The right LED will display “E” if no saved mode exists in this slot, otherwise “F”.  Use a short press with the right button to recall these settings.  Use a long press with the right button to save the current switch setup to this flash slot.  When using IR, the sequence is (Output type select “F”, input select 0-15).  Use input select codes 0-7 to recall flash modes 1-8, and input select codes 8-15 to save to flash modes 1-8.

The IR remote, RS-232, and Ethernet interfaces can select outputs and inputs directly.  If using the Ethernet or RS-232 interface, please refer to the programming guide on our web site.   The IR interface is explained later in this document.

The table below explains what the LED displays indicate.  Note that generally the left two digits display the selected output and the right two digits display the selected input.  When selecting an output greater than 9, the letter will flash for 1 second and then the “1” will be displayed.  When “0” is displayed as the selected input that output is powered off.

| Left LED Display (selected output) | Right LED Display (selected input) | Description |
| --- | --- | --- |
| U 1-16 | 0-16 | Component video control |
| F 1-8 | E/F | Flash mode slot 1-8. If right LED shows E, then slot is empty. If right LED shows F, then slot is active. |
| CF | C-U, O/F | Config menu, please see setting below |
| SZ | 0-41 | Zerokey programming mode, please see Zerokey section |

Right LED display for configuration menu:

- C  	Clear memory.  Press the right front panel key and hold for 6 seconds to reset all Zerokey IR codes to factory defaults and put the switch into basic mode.

- L	LED OFF.  Press and hold the right switch to black out the LED display.  Any button will restore.

- P	Power OFF.  Press and hold the right switch to turn off CMX.  Any button will restore power.

- U O/F	Reserved, has no effect

- 1 O/F	Debug control (do not use).  Press & hold the right switch to turn on/off

- 2 O/F	Debug control (do not use).  Press & hold the right switch to turn on/off

- 3 O/F	Power up default.  When this control is switched from F to O, it will save the current state of all outputs to flash memory.  At power up if this control is O, the saved state will be loaded at power up.  If it is F, all outputs will be disabled on power up.

IR remote interface

The VMX99 IR interface consists of 43 discrete IR codes.  These codes are stored in internal flash memory.  When the VMX99 detects a transmitted IR code which matches one or more of the codes in memory, it performs the action dictated by that code.  The table below shows what action each code performs.  The factory defaults for all 43 codes can be obtained in CCF format from our web site.  If you have a fully programmable remote, you can load these codes into the remote.  If your remote does not support loading hex IR codes, then you will need to program new codes into the VMX99.  See the section on programming the ZeroKey interface for instructions.

| IR code | Action performed |
| --- | --- |
| 0 | Selects the output type “U” (component video) |
| 1 | Reserved |
| 2 | Reserved |
| 3 | Reserved |
| 4 | Reserved |
| 5 | Reserved |
| 6 | Reserved |
| 7 | Selects the output type “CF” (config menu) |
| 8-23 | Specific output select 1-16. The output selected is the (IR code – 7) |
| 24-41 | Input select 0-16. The input selected is the (IR code – 24) |
| 42 | Selects the output type “F” (flash modes) |

The sequence for switching an output requires sending 3 IR codes.  First send the code to select the output type (code 0-7), then the code to select the specific output (code 8-23), then the code to select the input (code 24 – 40).   For example (in advanced mode), to switch component video output 3 to input 5 send codes 0, then 10, then 29.

Special considerations:

- Set ZeroKey mode cannot be accessed by IR or RS-232.  This must be done by front panel buttons if necessary (one time only).

- Config menu:  The specific output select code is not used (ignored).  The input select codes work as follows.

| Input select (IR code) | Action |
| --- | --- |
| 1 (25) | Turns LED OFF |
| 9 (33) | Turns LED ON |
| 2 (26) | Powers OFF VMX |
| 10 (27) | Powers ON VMX |
| other | All other codes should not be used when in config menu |

Programming the ZeroKey remote interface

The remote control interface can use ZeroKey infrared code cloning to map any remote control key to each of the 43 IR control codes.  Depending on how you are using VMX99, you may not need to program every IR code.  For example, if you are using VMX99 without HDMI, you only need to program IR codes 0, 3, 4, 8-24, and 24-40 (36 total codes).

Before starting, you will need a remote with as many unique “keys” as codes you wish to program.  Keep in mind; most universal remotes have 5-6 “pages” of codes.  For example, when you select “VCR” control the numeric keys on the remote have different IR codes than when you select “DVD” on the remote.

Here are the steps to program the VMX99 Zerokey control codes for your remote.

- Toggle the left button until “SZ” is displayed on the left LED

- The right LED will display a number 0-40.  This corresponds to the IR code selected.

- Toggle the right button to the IR code you wish to program

- Press and hold the right button for 6 seconds.  When released the right LED should start to flash.

- Press the remote button you wish to map to this IR code

- The right LED should stop flashing, indicating the code was stored.

- The CMX may switch at this point if programming IR codes 0-7.  This is normal, just toggle back to the “SZ” mode.

- Repeat for all other IR codes you wish to use.

- When done, test every IR code.  If one or more do not work repeat the programming for those codes (sometimes interference will cause the code learn to fail).  If you cannot get a specific code to work after multiple attempts, try using a different remote key.

For additional examples of programming the ZeroKey IR interface, please refer to the quick start guide.

ZeroKey technology is compatible with virtually all remote controls, and does not interfere with the normal operation of the remote.  You may reset all the ZeroKey codes to the factory defaults by selecting “CFC” and holding the input button for 6 seconds

ZeroKey programming is stored in flash memory, and you will not need to reprogram even if you lose power.  If you buy a new remote in the future, don't worry, the VMX99 can be reprogrammed as much as you like.

Specifications

- 27 High bandwidth analog inputs  (used for component video and composite video)
    75Ohm terminated at all frequencies
    Gold plated matched low impedance RCA jacks
    1Vpp nominal input swing (Ypbpr) or 1.3Vpp (Cvbs)   4Vpp maximum  (DC invariant)
    DC rejection via capacitive coupling
    Crosstalk < -80 dB

- 27 High bandwidth analog outputs  (used for component video and composite video)
   Source impedance 75 Ohms
   Gold plated matched low impedance RCA jacks
   Selectable AC/DC coupling
   All output sets individually controllable/switchable
   High performance amplifier capable of driving over 300ft of coax cable per output
   2V/V (video) gain to properly drive output load termination (zero effective gain at display)
   -3dB bandwidth > 100 Mhz

- IR receiver (front)

- RS-232 control input (3.5mm female stereo)

- Ethernet control input (RJ45)

- Internal design 
    Flash programmable micro controller enables ZeroKey remote technology
    Ultra wide bandwidth integrated buffered t-switch matrix prevents crosstalk or signal distortion
    Independent professional quality video amplifiers for best performance
    High reliability capacitors and self resetting fuse protection for long life
    Designed and manufactured in the USA

- Housing 
    Black steel rack mountable housing   17" x 13" x 1.75"   (1 RU)